CLIL PARTE 2
STEPPER MOTOR

Read the attached code and then create the circuit capable of realizing it in the wokwi simulator.

// test a stepper motor with a Pololu A4988 driver board or equivalent

// this version uses millis() to manage timing rather than delay()

// and the movement is determined by a pair of momentary push switches (push button)

// press one and it turns CW, press the other and it turns CCW

byte directionPin = 9;

byte stepPin = 8;

byte buttonCWpin = 10;

byte buttonCCWpin = 11;

boolean buttonCWpressed = false;

boolean buttonCCWpressed = false;

byte ledPin = 13;

unsigned long curMillis;

unsigned long prevStepMillis = 0;

unsigned long millisBetweenSteps = 25; // milliseconds

void setup() {

Serial.begin(9600);

Serial.println("Starting Stepper Demo with millis()");

pinMode(directionPin, OUTPUT);

pinMode(stepPin, OUTPUT);

pinMode(ledPin, OUTPUT);

pinMode(buttonCWpin, INPUT_PULLUP);

pinMode(buttonCCWpin, INPUT_PULLUP);

}

void loop() {

curMillis = millis();

readButtons();

actOnButtons();

}

void readButtons() {

buttonCCWpressed = false;

buttonCWpressed = false;

if (digitalRead(buttonCWpin) == LOW) {

buttonCWpressed = true;

}

if (digitalRead(buttonCCWpin) == LOW) {

buttonCCWpressed = true;

}

}

void actOnButtons() {

if (buttonCWpressed == true) {

digitalWrite(directionPin, LOW);

singleStep();

}

if (buttonCCWpressed == true) {

digitalWrite(directionPin, HIGH);

singleStep();

}

}

void singleStep() {

if (curMillis - prevStepMillis >= millisBetweenSteps) {

// next 2 lines changed 28 Nov 2018

//prevStepMillis += millisBetweenSteps;

prevStepMillis = curMillis;

digitalWrite(stepPin, HIGH);

digitalWrite(stepPin, LOW);

}

}

Add the WOKWY schema function in the box below and any explanations.

Briefly answer the following questions

- What are the main parts of a stepper motor?

- Why are powered stepper motors hot even if the shaft is not rotating?

- What are the main differences between bipolar and unipolar steppers?

- What type of stepper motor provides the most torque at the same speed?

- What type of engine is the 28BYJ-48?

- How much current does a small stepper motor typically consume?

- What is the typical working voltage of stepper motors?

- What is a "driver"?

- What is meant by "microstepping"?

- How do you choose a stepper motor?

- What does the abbreviation NEMA 23 mean?

- With a 1/16 microsteps what angular precision is obtained?

- What is the maximum speed of a stepper motor?

- What torque does a small NEMA 17 motor typically have?

- What does the Arduino code in “sample 1” do?

VIDEO ABOUT STEPPER MOTOR

https://howtomechatronics.com/tutorials/arduino/how-to-control-stepper-motor-with-a4988-driver-and-arduino/

https://howtomechatronics.com/tutorials/arduino/stepper-motors-and-arduino-the-ultimate-guide/