MISURA NUNERO GIRI ALBERI

32x14mm

Cuscinetto  693ZZ
8x3x4  mm

### Project#11 Infrared speed sensor module (applied on DIY anemometer)

Mini Motor Speed Sensor -A from Amazon Mini Motor Speed Sensor -B from Amazon - Motor speed sensor or work piece counting - Using infrared detection, strong anti-jamming capability - Schmidt trigger, high sensitivity, stable performance - 5V Working voltage, black line output low level and white lines output high level - 74HC14 Hex inverting high performance Schmitt trigger chip - Photo interrupter: MOCH25A; slotted width : 1 cm LED-D2 indicates the presence of voltage applied; the LED-D1 monitors operation of the phototransistor. Optocoupler, U1 - MOCH25A, where the wheel encoder disc is placed. One side was an infrared led, and the other was a phototransistor. If no object block, the phototransistor would conduct; when something blocked the light falling on the transistor it wouldn’t conduct. Chip U2 - 74HC14 Hex inverting. When there is no object between the photo interrupter will be logical “0”, U2 generates a logical "1" and the LED-D1 is off. During the rotation, the object will block light output from optocoupler will be a logical "1" and U2 is logical “0”, simultaneously, the LED-D1 is on . Wheel Encoder Disc (Encoder resolution: 20 holes / disc): Code: 1 2 3 4 5 6 7 8 9 10 11 12 13 14 15 16 17 18 19 20 21 22 23 24 25 26 27 28 29 30 31 32 33 34 35 36 37 38 39 40 41 42 43 44 45 46 47 48 49; int encoder_pin = 2; // The pin the encoder is connected unsigned int rpm; // rpm reading volatile byte pulses; // number of pulses unsigned long timeold; // The number of pulses per revolution // depends on your index disc!! unsigned int pulsesperturn = 20; void counter() { //Update count pulses++; } void setup() { Serial.begin(9600); //Use statusPin to flash along with interrupts pinMode(encoder_pin, INPUT); //Interrupt 0 is digital pin 2, so that is where the IR detector is connected //Triggers on FALLING (change from HIGH to LOW) attachInterrupt(0, counter, FALLING); // Initialize pulses = 0; rpm = 0; timeold = 0; } void loop() { if (millis() - timeold >= 1000){ /*Uptade every one second, this will be equal to reading frecuency (Hz).*/ //Don't process interrupts during calculations detachInterrupt(0); //Note that this would be 60*1000/(millis() - timeold)*pulses if the interrupt //happened once per revolution rpm = (60 * 1000 / pulsesperturn )/ (millis() - timeold)* pulses; timeold = millis(); pulses = 0; //Write it out to serial port Serial.print("RPM = "); Serial.println(rpm,DEC); //Restart the interrupt processing attachInterrupt(0, counter, FALLING); } }. Output result as below; it's close to the spec of motor's RPM value =240 @6V NOTE: Consider Bad Pulses described here Add 100nF (104) metallized polyester Capacitor to the encoder, Code: 1 2 3 4 5 6 7 8 9 10 11 12 13 14 15 16 17 18 19 20 21 22 23 24 25 26 27 28 29 30 31 32 33 34; // https://www.youtube.com/watch?v=9848QETGd_k // Using HC-020K Photoelectric encoder on the Dual Axis Reducer Motor // with 100nF capacitor on the encoder #include <TimerOne.h> // library: https://code.google.com/archive/p/arduino-timerone/downloads int encoder_pin = 2; unsigned int counter=0; void setup() { Serial.begin(9600); pinMode(encoder_pin, INPUT); Timer1.initialize(1000000); // set for 1 sec attachInterrupt(0,do_count,RISING); Timer1.attachInterrupt(timerIsr); } void loop() { // put your main code here, to run repeatedly: } void do_count() { counter++; } void timerIsr() { Timer1.detachInterrupt(); Serial.print("Motor Speed: "); int rotation = (counter/20)*60; Serial.print(rotation, DEC); Serial.println(" RPM"); counter=0; Timer1.attachInterrupt(timerIsr); }. Result: LM393 Comparator IR Speed Sensor Module (FC-03) from Amazon Working Voltage : DC 3.3-5V Main Chip : LM393 Slotted Width : 5mm ITR9608 photoelectric switch (not sure, I guessed) Pins Connection: VCC: positive power supply 3.3-5V; GND: Ground; DO: Output frequency pulses; AO: analog output, real-time output voltage signal. (not useful) The LED (left) indicates the presence of voltage applied. When there is no object between the photo interrupter, the LED (right) is ON; otherwise is OFF. If slot unobstructed, DO output low; otherwise, DO output high. Using the same code above, output as below: Note: I use 3.3V as VCC to get below result; if 5V is used, only get half RPM readings. # HC-020K Double Speed Measuring Module with Photoelectric Encoders from DX.com or Amazon - Module Working Voltage: 4.5-5.5V - Launch Tube Pressure Drop: Vf=1.6V - Launch Tube Current: If<20mA - Signal output: ONE line; TT power level; - Slotted Width : 6mm - Chip: LM393 # Slotted LM393 Beam Infrared Light Counter Photoelectric Sensor Module from Amazon Chip: LM393 Power : DC5V; Slotted Width : 10mm # FC-33 Electric Motor Speed Sensor Module from DX.com Working voltage: 3.3~5V Chip: LM393 Slotted Width : 10mm The module can apply to DIY anemometer, it measures the wind flow velocity. construction detail as https://www.instructables.com/id/DIY-Anemometer-and-Windvane-for-Standalone-Weather/